| HuangHai Electric Pickup |  |
| --- | --- |
| Security Access Algorithm |  |

Document status： [√ ] Draft [ ]Published [ ] Modification

Created by:

| Created by： |  | Date： | 2023-01-18 |
| --- | --- | --- | --- |
| Verified by： |  | Date： |  |
| Approved by： |  | Date： |  |

Statement of copyrights

This technical document belongs to the property of HuangHai. The use, copy and publish of any information included in this document shall be permitted by HuangHai.

Revision History

| Version | Description | Date | Modified by |
| --- | --- | --- | --- |
| 1.0 | Initial version | 2023-01-18 |  |

Contents

1	General	1

2	Security Access Algorithm	1

2.1	Variables Declaration	1

2.2	Flow Chart	1

2.3	Algorithm Instances	2

- General

This document is intended to establish the security access algorithm and instruction for service $27.

The algorithm used as key for service $27 is defined in this document.

- Security Access Algorithm

  - Variables Declaration

The variables and data are described as follow:

Char Seed[4] = {S1, S2, S3, S4} ，Seed array

Char Cal[4] = {C1, C2, C3, C4} ，Calculation array

Char Key[4] = {0x00, 0x00, 0x00, 0x00} ，Key array

Char Xor [4] = {X1, X2, X3, X4} ，XOR array

Table 1 defines the XOR array for AVM.

| ECU | X1 | X2 | X3 | X4 |
| --- | --- | --- | --- | --- |
| AVM | E4 | 2F | 45 | 92 |

- Table 1   AVM XOR[] array

  - Flow Chart

2.2.1 Security level1（0x01/0x02）

Figure 1  Level 1 flow chart

2.2.2 Security level FBL（0x09/0x0A）

Figure 2  Level FBL flow chart

  - Algorithm Instances

AVM（level 1）

Xor： 0xE4  0x2F  0X45  0X92

Seed：0x12  0x16  0XE1  0x78

Key： 0xE0  0xA9  0xCE  0x64

AVM（level FBL）

Xor：  0xE4  0x2F  0X45  0X92

Seed：0x12  0x16  0XE1  0x78

Key： 0x49  0x6A  0x3E  0x0F